# Application 10: Connection to RS-422 device

- The controller scans & analyzes the onboard D/I, if the D/I states match with the alarm states, the onboard D/O will drive the alarm device for alarm or safety control.

- The 7522A can connect to one RS-232 device and one RS-422 device. The 7522A also provide 4 channels of D/I & 4 channels of D/O onboard. These D/I can be used to link to photo sensors or limit switchs. These D/O can be used to drive relay or led for alarm state control.

Refer to 7522AP10.c for source code of firmware

Application 11: 7520 with Temperature Value Display

- The 7521D can be used to replace 7520 & show the local temperature T0 ~ T15 without any help of host-PC. The 7521D will send command to 7018 to read & save T0 ~ T15. The 5-digit LEDs will show channel number first, then show the related temperature value. So the operator can read the local temperature, T0 ~ T15, directly without the help of host-PC.

- The original program in host-PC designed for 7520 version can be executed for 7521D without any modifications.

- Refer to 7521AP11.c for source code of firmware

Original program for 7520 without modification

Application 12: RS-485 to RS-232/485/422 Device Server

- The 7521 family can be used as RS-485 to RS-232/485/422 device server

- Install the 7521 to RS-485 network first

- Run Plug&Play Utility will virtualize the 7521 to act as COM3 of host-PC. Then any program call MS-COMM standard to access COM3 will refer to HP34401A-1.

- Install 7522 to the RS-485 network & run Plug&Play Utility will virtualize 7522 to COM4 & COM5 of host-PC. Then program in host-PC can access HP34401A-2 through COM4 & access HP34401A-3 through COM5.

- Install 7522A/7523/7524/7527 one by one to virtualize COM6 to COM21

- After COM3 to COM21 are virtualized, the conventional package designed for MS-COMM standard can access these remote devices without any modification.

- The max. number of COM port = 255

- Refer to “Communication Controller User’s Manual” for more information.

The mapping Table of VxCOM is given as following:

| 7521 family | Module address of 7521 family | VxCOM in host-PC | Remote Devices |
| --- | --- | --- | --- |
| 7521 | 0x01 | COM 3 | HP34401A-1 |
| 7522 | 0x02 | COM 4 | HP34401A-2 |
|  |  | COM 5 | HP34401A-3 |
| 7522A | 0X04 | COM 6 | HP34401A-4 |
|  |  | COM 7 | FX2 |
| 7523 | 0X06 | COM 8 | HP34401A-5 |
|  |  | COM 9 | HP34401A-6 |
|  |  | COM 10 | HP34401A-7 |
| 7524 | 0X09 | COM 11 | HP34401A-8 |
|  |  | COM 12 | HP34401A-9 |
|  |  | COM 13 | HP34401A-10 |
|  |  | COM 14 | HP34401A-11 |
| 7527 | 0X0D | COM 15 | RS-232 device-1 |
|  |  | COM 16 | RS-232 device-2 |
|  |  | COM 17 | RS-232 device-3 |
|  |  | COM 18 | RS-232 device-4 |
|  |  | COM 19 | RS-232 device-5 |
|  |  | COM 20 | RS-232 device-6 |
|  |  | COM 21 | RS-232 device-7 |

Application 1: Single-channel RS-232 Device Server

- 7188E1 is designed for single-channel RS-232 device server. It can link a general RS-232 device to Ethernet 10M network.

- Install the first 7188E1 to Ethernet 10M network first

- Run Plug&Play Utility in host-PC will virtualize this 7188E1 to act as COM 3 of host-PC. Then any program call MS-COMM standard to access COM 3 will refer to HP34401A-1. So the original application program designed for HP34401A can still work if change COM1/COM2 to COM3. In general, this change can be done by configuration utility. The original source codes can work perfect without any modification.

- Install the second 7188E1 to Ethernet 10M network & run Plug&Play Utility will virtualize the second 7188E1 to COM 4 of host-PC.

- Install the other 7188E1 one by one to virtualize COM Y of host-PC

- After COM 3 to COM Y are virtualized, the conventional package designed for MS-COMM standard can access these remote devices without any modification.

- The max. number of COM port = 255

- Refer to “Communication Controller User’s Manual” for more information.

User’s original application program designed for HP34401A can be work without any modification

Application 2: 7000 Series Device Server

- Refer to Application 1 for more information.

- 7188E2 can link 7000 series modules to Ethernet 10M network.

- 7188E2 can be virtualized as COM 3 & COM 4 of host-PC. COM 3=RS-232 & COM 4 = RS-485 = link to 7000 series modules.

- The original package designed for 7000 series modules can work perfect through COM4 without any modification.

- The max. number of COM port = 255

- Refer to “Communication Controller User’s Manual” for more information.

Application 3: RS-422 Device Server

- Refer to Application 1 for more information.

- 7188E3 can link RS-422 device to Ethernet 10M network.

- 7188E3 can be virtualized as COM 3 , COM 4 & COM 5 of host-PC. COM 3=RS-232 & COM 4=RS-485, COM 5 = link to RS-422 device.

- 7188E3 equips 4 channels of D/I & 4 channels of D/O. These D/I can be linking to photo sensor or limit switch sensor. These D/O can directly drive relay or led for emergency control.

- The max. number of COM port = 255

- The device server does not support D/I & D/O of 188E3. But the addressable RS-422 converter & embedded communication controller can support these D/I & D/O. Refer to “Communication Controller User’s Manual” for more information.

Application 4: Modem Device Server

- Refer to Application 1 for more information.

- 7188E4 can link modem device to Ethernet 10M network.

- 7188E4 can be virtualized as COM 3 , COM 4 & COM 5 of host-PC. COM 3=RS-232 & COM 4=RS-485, COM 5 = link to RS-422 device.

- The 7188E3 equips 4 channels of D/I & 4 channels of D/O. These D/I can be linking to photo sensor or limit switch sensor. These D/O can directly drive relay or led for emergency control.

- The max. number of COM port = 255

- The device server does not support D/I & D/O of 188E3. But the addressable RS-422 converter & embedded communication controller can support these D/I & D/O. Refer to “Communication Controller User’s Manual” for more information.

Application 5: N-port RS-232 Device Server

- Refer to Application 1 for more information.

- 7188E5 can link 4 RS-232 devices to Ethernet 10M network.

- 7188E5 can be virtualized as COM 3/4/5/6/7 of host-PC. COM 4=RS-485, The other 4 COMs = link to RS-232 device.

- All RS-232 port supports RXD, TXD, RTS, CTS, RTS & GND

- The COM 2 of 7188E5 is empty. But it also mapping to COM 4 of host-PC. So any program in host-PC accesses to COM 4 will not get any response.

- The max. number of COM port = 255

- Refer to “Communication Controller User’s Manual” for more information.

Application 6: 3-wire RS-232 Device Server

- Refer to Application 1 for more information.

- 7188E8 can link 7 RS-232 devices to Ethernet 10M network.

- 7188E8 can be virtualized as COM 3/4/5/6/7/8/9/10 of host-PC. COM 4=RS-485, The other 7 COMs = link to RS-232 device.

- COM1 supports RXD, TXD, RTS, CTS, RTS & GND (5-wire)

- The other RS-232 ports support RXD, TXD & GND (3-wire)

- The COM 2 of 7188E8 is empty. But it also mapping to COM 4 of host-PC. So any program in host-PC accesses to COM 4 will not get any response.

- The max. number of COM port = 255

- Refer to “Communication Controller User’s Manual” for more information.

Application 7: 7188EN family Device Server

- Refer to Application 1 for more information.

- Install 7188E1 first, then 7188E2/3/4/5/8 one by one

- The max. number of COM port = 255

- Refer to “Communication Controller User’s Manual” for more information.

Application 8: Mixed-Network Device Server

- Refer to Application 1 for more information.

- In this application, 7521 family link through RS-485 network. 7188EN family link to the host-PC through Ethernet 10 BaseT. Without the help of Plug & Play Utility, the remote RS-232/485/422 devices are mapping to COM3/4/…/N of host-PC. The original program can access these devices attached in RS-485 network or Ethernet 10 BaseT in the same way.

- The max. number of COM port = 255

- Refer to “Communication Controller User’s Manual” for more information.

## Software Block Diagram

The software environment of 7188E family can be divided into two parts as (1) software for host-PC & (2) firmware inside controller.

(1) software for host-PC:  provide related software for user to link to COM port or Ethernet port easily. Refer to Sec. 3.2 for more information.

(2) firmware inside controller: provide MiniOS7, TCP/IP & Web server library & communication controller firmware for user to write application program in 7188E family. Refer to Sec. 3.3 for more information.

## Software for Host-PC

## Firmware Inside Controller