Using CEC-488 boards with Borland Delphi 2.0

------------------------------------------------

1) Add the file IEEEDEL.PAS to your Delphi project.

2) In any module where you wish to use the CEC-488 routines,

add the line:

uses ieeedel;

3) Call the routines just as documented throughout the CEC-488

manual for the Pascal language.

For example:

initialize (21,0);

send (16, '*IDN?', status);

enter (r, 80, len, 16, status);

You also need the file IEEE_32M.DLL, which is automatically

installed as part of running SETUP on a Windows system.