Turbo BASIC and PC-488

Turbo BASIC from Borland International can be used with the

PC-488 interface card.  The files "TB488.BAS", "TBXMIT.COM",

"TBRECV.COM", "TBSEND.COM", and "TBENTER.COM" from the

Programming and Reference disk are all that is required.

Turbo BASIC provides a CALL ABSOLUTE statement which works for

any subroutines which use only integer arguments.  This leaves

only the TRANSMIT, RECEIVE, SEND, and ENTER routines to be

taken care of.

Simply merge or include the file "TB488.BAS" into your Turbo

BASIC program.  This file contains definitions for the

routines TRANSMIT, RECEIVE, SEND, and ENTER.

A FIND routine is also provided to search the PC's memory for

the PC-488 board.  This eliminates the need for knowing the

PC-488's segment address in advance.

* Turbo BASIC is a registered trademark of Borland International

Subroutine calling sequences:

CALL FIND

DEF SEG=PC488SEG%

'

'  NOTE: It is important to call FIND once at the start of

'        your program.  This determines the address of the

'        PC-488 board.  Follow the call with a "DEF SEG".

'

INIT=0

CALL ABSOLUTE INIT (ADDR%,LEVEL%)

CALL TRANSMIT (S$,STATUS%)

CALL RECEIVE (R$,LENGTH%,STATUS%)

CALL SEND (ADDR%,S$,STATUS%)

CALL ENTER (R$,LENGTH%,ADDR%,STATUS%)

SPOLL=12

CALL ABSOLUTE SPOLL (ADDR%,POLL%,STATUS%)

PPOLL=15

CALL ABSOLUTE PPOLL (POLL%)

TARRAY=200

CALL ABSOLUTE TARRAY (SEGMENT%,OFS%,COUNT%,EOI%,STATUS%)

RARRAY=203

CALL ABSOLUTE RARRAY (SEGMENT%,OFS%,COUNT%,LENGTH%,STATUS%)

DMA2=206

CALL ABSOLUTE DMA2 (SEGMENT%,OFS%,COUNT%,MODE%,STATUS%)

Example (for an HP 7470 plotter):

$INCLUDE "TB488.BAS"

CALL FIND            ' search for PC-488 board

DEF SEG=PC488SEG%    ' set segment as determined by FIND

INIT=0 : SPOLL=12    ' PC-488 subroutine offsets

MY.ADDR%=21 : LEVEL%=0                       ' initialize

CALL ABSOLUTE INIT (MY.ADDR%,LEVEL%)

ADDR%=5

CALL ABSOLUTE SPOLL (ADDR%,POLL%,STATUS%)    ' do a serial poll

PRINT POLL%

CALL SEND (5,"OI",STATUS%)                   ' send some data

R$=SPACE$(30)                                ' read some data

CALL ENTER (R$,LENGTH%,5,STATUS%)

Example using RARRAY:

$INCLUDE "TB488.BAS"

DIM ARRAY%(100)

CALL FIND

DEF SEG=PC488SEG%

CALL TRANSMIT ("MLA TALK 3",STATUS%)         ' set talker/listener

RARRAY=203

SEGMENT%=VARSEG(ARRAY%(0))           ' get segment for ARRAY%

OFS%=VARPTR(ARRAY%(0))               ' get offset for ARRAY%

COUNT%=200

CALL ABSOLUTE RARRAY (SEGMENT%,OFS%,COUNT%,LENGTH%,STATUS%)